### 1. Mục đích bài viết

- This article will guide in detail the configuration of the Palo Alto Networks firewall device to allow users within the LAN to access over the Internet..

### 2. Network diagram, configuration situation and steps to be taken.

#### 2.1 Network digram :

#### 2.2 Detailed network diagram :

- As the diagram, the Palo Alto firewall device will be connected to the internet in port 1 with a static IP of 192.168.1.202/24 and point to the gateway that is the address of the network 192.168.1.1/24.

- On the inside of Palo Alto is the intranet layer with IP 192.168.10.1/24 set to port 2. On port 2 is configured DHCP server to allocate IP for devices accessing it.

- Finally, a Laptop device is connected to port 2 via a network cable and receives IP 192.168.10.201 from the DHCP server on port 2.

#### 2.3 Configuration steps :

- Connect to the admin site of the firewall device.

- Create zone.

- Create Interface Mgmt Profile.

- Network port configuration.

- Create Virtual Router.

- DHCP Server configuration.

- Create NAT policy.

- Create Security Policy Rule.

- Result

### 3. Configuration guide.

#### 3.1 Connect to the admin site of the firewall device .

- We will connect to the firewall administration page using a network cable connecting the computer to the MGMT port of the Palo Alto firewall.

- Open the browser and access by the link https://192.168.1.1. The default account and password for the Palo Alto firewall are admin – admin.

#### 3.2 Create zone

- We will create two zones, WAN and LAN.

- To create a WAN zone go to Network > Zones > click Add, enter the following information :

  - Name : WAN

  - Type : Layer 3

- Click OK to save.

- Similarly click Add to create a LAN zone with the following parameters :

  - Name : LAN

  - Type : Layer 3

- Click OK to save.

#### 3.3 Create Interface Mgmt File

- The purpose of creating Interface Mgmt profile is to open some essential services for any network port such as HTTPS, Ping …

- Here we will create an Interface Mgmt Profile that allows HTTPS, Ping, SSH, Reponse Pages services for 1/2 ethernet LAN port so we can ping, access the admin site on this port without connecting by Mgmt port.

- To create Interface Mgmt Profile go to Network> Interface Mgmt> click Add and enter the following parameters :

  - Name : ping-reponse-pages

  - Administrative Management Services : select HTTPS, SSH.

  - Network Services : select ping và reponse pages.

- Click OK to save.

#### 3.4 Network port configuration

- To configure ethernet1/1 network port go to Network > Interfaces > click on the network port name.

- In the Config tab, configure the following parameters :

  - Interface type : select Layer 3

  - Security Zone : select WAN

- In the IPv4 tab, configure according to the following parameters :

  - Type : select Static

  - Click Add và type IP address 192.168.1.202/24.

- Click OK to save.

- Similar to ethernet1/1 port, to configure ethernet1/2 port click on the port name.

- In the Config tab, configure according to the following parameters :

  - Interface Type : Layer 3

  - Security Zone : LAN

- In the IPv4 tab, we configure the following parameters :

  - Type : Static

  - Click Add và type IP 192.168.10.1/24

- In the Advanced tab, configure the following :

  - In Other Info > Management Profile select the ping-reponse-pages we just created in the previous section.

#### 3.5 Create Virtual Router

- To create a Virtual Router go to Network > Virtual Router > Click Add.

- In the Router Settings tab, configure according to the following parameters :

  - Name : VR1

  - In the General panel, click Add and add 2 port ethernet1/1 and Ethernet1/2.

- On the Static Routes tab, click Add and configure according to the following parameters :

  - Name : default-route

  - Destination : 0.0.0.0/0

  - Interface : ethernet1/1

  - Next Hop : IP Address and enter 192.168.1.1 in the box below.

- Click OK two time to save.

#### 3.6 DHCP Server configuration

- To configure DHCP Server go to Network > DHCP > click Add.

- In the Lease tab, configure with the following parameters :

  - Interface : select ethernet1/2

  - Mode : enable

  - In the IP Pools table click Add and fill in the IP range that will be allocated as 192.168.10.200-192.168.10.230.

- In the Options tab, configure with the following parameters :

  - Gateway : 192.168.10.1

  - Subnet Mask : 255.255.255.0

  - Primary DNS : 8.8.8.8

  - Secondary DNS : 8.8.4.4

- Click OK to save.

#### 3.7 Create NAT Policy

- To create NAT Policy go to Policies > NAT > Click Add.

- In the General tab, configure according to the following parameters :

  - Name : LAN_TO_WAN

  - NAT Type : ipv4

- In the Original Packet tab, configure according to the following parameters :

  - Source Zone : LAN

  - Destination Zone : WAN

  - Destination Interface : ethernet 1/1

- In the Translated Packet tab > Source Address Translation, configure the following parameters :

  - Translation Type : Dynamic IP and Port

  - Address Type : Interface Address

  - Interface : ethernet1/1

  - IP Address : 192.168.1.202/24 (Note that this location must be selected from the drop down list, not entered manually)

#### 3.8 Create Security Policy Rule

- To create Policies > Security > Click Add.

- In the General tab, configure according to the following parameters :

  - Name : Access_Internet

  - Rule Type : universal (default)

- On the Source tab, select LAN in Source Zone.

- On the Destination tab, choose WAN in Destinatoin Zone.

- On the Application tab, select Any.

- On the Service / URL Category tab, select any.

- In the Action tab, configure the following :

  - Action Setting : Allow

  - Log Setting : Log at Session End.

- Click OK to save.